Moments Calculations

- A balanced see saw pivotted in the centre has a boy of mass 42kg on one side and a girl of mass 35kg on the other. If the boy is 1.6m from the pivot, how far is the girl?

- A balanced see saw pivotted in the centre has a boy of mass 24kg on one side and a girl of mass 18kg on the other. If the boy is 1.2m from the pivot, how far is the girl?

- A balanced see saw pivotted in the centre has a boy of mass 32kg on one side and a girl on the other. If the boy is 1.6m from the pivot, and the girl is 2.1m, what is the mass of the girl?

- A balanced see saw pivotted in the centre has a boy of mass 40kg on one side and a girl on the other. If the boy is 1.8m from the pivot, and the girl is 0.6m, what is the mass of the girl?

- A see saw pivotted in the centre has a boy of mass 82kg on one side and a girl of mass 24kg on the other. If the boy is 1.2m from the pivot, and the girl is 1.5m, how far from the pivot does a second girl of mass 32kg have to sit to balance the see saw?

- A see saw pivotted in the centre has a boy of mass 94kg on one side and a girl of mass 40kg on the other. If the boy is 1.6m from the pivot, and the girl is 2.0m, how far from the pivot does a second girl of mass 32kg have to sit to balance the see saw?

- A 2m balanced see saw of mass 15kg is not pivotted at its centre. It has a child of mass 20kg at one end and another child of mass 10kg at the other. Draw a diagram of the situation. Label the distance from the pivot to the centre of mass d. Determine the distance to each end in terms of d. Hence or otherwise determine a value for d.

- A 1.6m balanced see saw of mass 10kg is not pivotted at its centre. It has a child of mass 30kg at one end and another child of mass 10kg at the other. Draw a diagram of the situation. Label the distance from the pivot to the centre of mass d. Determine the distance to each end in terms of d. Hence or otherwise determine a value for d.

- A 1m balanced see saw of mass 5kg is not pivotted at its centre. It has a child of mass 24kg at one end and another child of mass 12kg at the other. Draw a diagram of the situation. Label the distance from the pivot to the centre of mass d. Determine the distance to each end in terms of d. Hence or otherwise determine a value for d.

- A 2.8m ladder leans against a wall so that the foot of the ladder is 0.9m away from the wall. The mass of the ladder is 12kg.

  - Sketch the situation

  - Label the reaction force of the floor

  - Label the reaction force of the wall

  - Label the frictional force of the wall

  - Determine the height the ladder goes up the wall

  - Determine the moments about the foot of the ladder caused by the weight of the ladder

  - By taking moments about the foot of the ladder, determine the reaction force of the wall

  - By taking moments about the top of the ladder, determine the reaction force of the floor

  - Hence or otherwise determine the friction at the wall

- A 3.4m ladder leans against a wall so that the foot of the ladder is 1.2m away from the wall. The mass of the ladder is 16kg.

  - Sketch the situation

  - Label the reaction force of the floor

  - Label the reaction force of the wall

  - Label the frictional force of the wall

  - Determine the height the ladder goes up the wall

  - Determine the moments about the foot of the ladder caused by the weight of the ladder

  - By taking moments about the foot of the ladder, determine the reaction force of the wall

  - By taking moments about the top of the ladder, determine the reaction force of the floor

  - Hence or otherwise determine the friction at the wall

- A 4.8m ladder leans against a wall so that the foot of the ladder is 1.8m away from the wall. The mass of the ladder is 20kg.

  - Sketch the situation

  - Label the reaction force of the floor

  - Label the reaction force of the wall

  - Label the frictional force of the wall

  - Determine the height the ladder goes up the wall

  - Determine the moments about the foot of the ladder caused by the weight of the ladder

  - By taking moments about the foot of the ladder, determine the reaction force of the wall

  - By taking moments about the top of the ladder, determine the reaction force of the floor

  - Hence or otherwise determine the friction at the wall

- A 1m plank of mass 14kg is supported at both ends. Draw a diagram of the situation, labelling the ends A and B.

  - By taking moments about A, determine the reaction force at B

  - Hence or otherwise, determine the reaction force at A

- A 3m plank of mass 24kg is supported at both ends. Draw a diagram of the situation, labelling the ends A and B.

  - By taking moments about A, determine the reaction force at B

  - Hence or otherwise, determine the reaction force at A

- A 1m plank of mass 15kg is supported by two bricks, one 15cm from end A and the other 25cm from end B. Draw a diagram of the situation.

  - By taking moments about the brick near A, determine the reaction force at B

  - Hence or otherwise, determine the reaction force at the brick near A

- A 1.8m plank of mass 12kg is supported by two bricks, one 24cm from end A and the other 38cm from end B. Draw a diagram of the situation.

  - By taking moments about the brick near A, determine the reaction force at B

  - Hence or otherwise, determine the reaction force at the brick near A

- A 2m plank of mass 7kg is pivotted 20cm from end A on which stands a small child. If the plank is balanced, determine the mass of the child. (Draw a diagram of the set up). Hence determine the reaction force at the pivot.

- A 3m plank is pivotted 50cm from end A on which stands a small child of mass 20kg. If the plank is balanced, determine the mass of the plank. (Draw a diagram of the set up)  Hence determine the reaction force at the pivot

- A 2m beam of mass 9kg is pivotted 20cm from end A on which stands a boy. If the beam is balanced when a child of mass 24kg stands  60cm from end B, determine the mass of the boy. (Draw a diagram of the set up)  Hence determine the reaction force at the pivot.

- A 2.4m beam of mass 12kg is pivotted 26cm from end A on which stands a girl. If the beam is balanced when a child of mass 18kg stands  36cm from end B, determine the mass of the girl. (Draw a diagram of the set up)  Hence determine the reaction force at the pivot.

- A 2m beam of mass 5kg is pivotted 35cm from end A on which stands a boy of mass 46kg. If the beam is balanced when a child of mass 24kg stands on the other side, determine the distance from end B that the second child must stand (Draw a diagram of the set up)  Hence determine the reaction force at the pivot.

- A 2.8m beam of mass 5kg is pivotted 42cm from end A on which stands a boy of mass 50kg. If the beam is balanced when a child of mass 24kg stands on the other side, determine the distance from end B that the second child must stand (Draw a diagram of the set up)  Hence determine the reaction force at the pivot.